NOTICE:  IMPORTANT REVISION TO D.O.T. DOCUMENTATION FOR HEDWIN CORP. WINPAK® PRODUCTS

Please review the attached information carefully.  This information is your official notification of the closure requirements for Hedwin WINPAK® packaging with UN marks applied in accordance with the Code of Federal Regulations Title 49, section 178.2 (c). The attached closure instructions cover every shipment of WINPAK® containers until further notice is sent to you by Hedwin.  In accordance with the new requirements effective October 1, 2010, shippers are required to retain a copy of this instruction in accordance with CFR Title 49, Section 173.22 (a) (4).  After you have read the attached document, please sign the statement below and fax to #410-243-0802 or mail to Hedwin Corporation, 1600 Roland Heights Avenue, Baltimore, Maryland 21211.

If you have any questions, please feel free to call 410-467-8209, extension 238 or 255.

I have received and read the attached DOT documentation.

_____________________________________

SIGNATURE

_____________________________________

NAME - PRINTED

_____________________________________

COMPANY NAME

Fax to 410-243-0802

SRN/cld

Spec. No.'s:  WRB7805, WRB7806, WRB7815, WRB7827, WRB7902,

WRB7925, WRB9805, WRB9806, WRF7824, WRS7470

DOT CLOSURE INSTRUCTIONS HEDWIN

20 LITER AND LESS RECTANGULAR

WINPAK® CONTAINERS

WINPAK® containers are self supporting, rectangular polyethylene containers designed for use in the transportation and storage of a wide variety of LIQUID products including hazardous chemicals.  It is the shipper's responsibility to determine if their chemical is allowed for shipment in the WINPAK® container as well as determine if the product is compatible with polyethylene.

WINPAK® containers are tested for compliance with the requirements of 49 CFR para. 178.600 and the containers carry the following dual marks:

# 3H1/Y1.8/100/YR*

# USA/M4151  1.1mm

# 3H1/Y1.3/100/YR*

# USA/M4151  1.1mm

*year of manufacture

1.3 mark is authorized for vented 70mm screwcaps.  Specific gravity of lading cannot exceed 1.3.

CLOSURE REQUIREMENTS

The container must be closed for shipment as listed below.  Use of nonspecified components will lead to non conformance with DOT marks.  If you are not the final assembler of the WINPAK® container, you are required to transfer this information to the party who will complete assembly of this package in accordance with 49 CFR para. 178.2.

| 70mm Closure Type and Dimensions | ## Source and Type | Torque Application Recommendation | Hedwin Wrench # |
| --- | --- | --- | --- |
| 70mm HDPE screwcap with EPDM gasket | Hedwin cap with EPDM gasket, Hedwin Part Number COM0892 | 70 in-lbs | TOL1458 see note 1 |
| 70mm HDPE screwcap with EPDM gasket | Rieke FS60-COM1688, Rieke FS70-COM0891 Rieke FS80-COM0895 with EPDM gasket | 120 in-lbs 120 in-lbs 120 in-lbs | TOL1486 see note 1 |
| 70mm HDPE screwcap with EPDM gasket, TE | Hedwin cap with EPDM gasket & TE band, Hedwin Part Number COM7025 | 120 in-lbs | TOL1486 see note 1 |
| 70mm HDPE screwcap with EPDM gasket, vented | Rieke FS70, PVTFE1GK98W EPDM gasket & microporous vent, see note 2 Hedwin Part Number COM1867 | 120 in-lbs | TOL1486 see note 1 |

| 18mm Closure Type and Dimensions | ## Source and Type | Torque Application Recommendation | Hedwin Wrench # |
| --- | --- | --- | --- |
| ### Screwcap Style 18mm polypropylene screwcap with polyethylene foam liner 18mm polypropylene Screwcap with f217 Triseal liner | Berry Plastics Hedwin Part Number COM6841 Berry Plastics Hedwin Part Number COM7950 | 10 in-lbs 10 in-lbs | TOL1443 see note 2 TOL1443 see note 2 |

Note 1:  Equivalent tools that apply appropriate torque can also be used.

Note 2:  Not authorized for air transport with vented caps.

Note 3:  18mm caps are torqued in place at Hedwin Corporation.  However, it is the responsibility of the shipper to verify the installation and correct torque of the 18mm closure after filling and prior to transport.

Applicable Specifications:  WRB7805, WRB7806, WRB7815, WRB7827, WRB7902,

WRB7925, WRB9805, WRB9806, WRF7824, WRS7470

Reference:  Lab Report #0274, #0275

Issued:  05/12/14

Hedwin Corporation

1600 Roland Heights Avenue

Baltimore, MD   21211

410-467-8209

www.hedwin.com